UA2000

Stereo Amplifier

| Product Description: The UA Series 2U professional amplifiers have been engineered for reliability where full power is required for extended times in fixed installations or portable applications.. The UA Series delivers extraordinary performance at unmatched value. Key Features and Benefits: - Housed in a rugged, all-steel 2U chassis - Rear panel selection switch for Stereo, Parallel and Bridged modes, including Low-cut filter, Ground lift and Limiter switches - Operates within 8Ω and 4Ω loads - Innovative heatsink with front to back air flow for efficient cooling - Variable fan speed is automatically adjusted by temperature - Electronically balanced XLR inputs - Touch proof binding post and Speakon outputs - Precision indented level controls, power switch, - Protect, Peak and Signal LEDs indicators for each channel - Amplifier protection includes short circuit, excessive temperature, and DC on outputs Front Panel: Rear Panel: Specifications. Rated Power (Stereo 8 Ω); 2 x 1000W. Rated Power (Stereo 4 Ω); 2 x 1600W. Rated Power (Bridge 16 Ω); 2000W. Rated Power (Bridge 8 Ω); 3200W. Rated Power (Parallel mono 8 Ω); 1000W. Rated Power (Parallel mono 4 Ω); 1600W. Input Sensitivity; 1V. S/N Ratio (A Weight); >108dB. Frequency Response (+0/-0.25dB); 20Hz-20kHz. Damping Factor; >800 @ 8ohm/100Hz. THD (10% Rated Power); <0.05%. Input Connector; 2 x XLR, 2 x 6.5mm TRS. Input Impedance; 20kΩ Balanced, 10kΩ Unbalanced. Output Connectors; 2 x NL4 + 4 x Binding post. Cooling; 4 x fans, airflow from front to back. Power Supply; 220V-230V 50/60Hz, 1500VA. Dimensions (W x D x H); 483mm (19”) x 452mm (17.8”) x 88mm (3.5”). Packaging Dimensions (1 piece/pack); 550mm (21.7”) x 540mm (21.3”) x 175mm (6.9”). Net Weight (1 piece/pack); 30kg (66.1lb). Gross Weight (1 piece/pack); 32kg (70.5lb). | Mouseover image to zoom Front View Back View Planform. |
| --- | --- |