UA2002

Stereo Amplifier

| Product Description: The UA Series 2U professional amplifiers have been engineered for reliability where full power is required for extended times in fixed installations or portable applications.. The UA Series delivers extraordinary performance at unmatched value. Key Features and Benefits: - Housed in a rugged, all-steel 2U chassis - Rear panel selection switch for Stereo, Parallel and Bridged modes, including Low-cut filter, Ground lift and Limiter switches - Operates within 4Ω and 2Ω loads - Innovative heatsink with front to back air flow for efficient cooling - Variable fan speed is automatically adjusted by temperature - Electronically balanced XLR inputs - Touch proof binding post and Speakon outputs - Precision indented level controls, power switch, - Protect, Peak and Signal LEDs indicators for each channel - Amplifier protection includes short circuit, excessive temperature, and DC on outputs Front Panel: Rear Panel: Specifications. Rated Power (Stereo 4Ω); 2 x 950W. Rated Power (Stereo 2Ω); 2 x 1500W. Rated Power (Bridge 8Ω); 1900W. Rated Power (Bridge 4Ω); 3000W. Rated Power (Parallel mono 4Ω); 1000W. Rated Power (Parallel mono 2Ω); 1650W. Input Sensitivity; 1V. S/N Ratio (A Weight); >108dB. Frequency Response (+0/-0.25dB); 20Hz-20kHz. Damping Factor; >800 @ 8ohm/100Hz. THD (10% Rated Power); <0.05%. Input Connector; 2 x XLR, 2 x 6.5mm TRS. Input Impedance; 20kΩ Balanced, 10kΩ Unbalanced. Output Connectors; 2 x NL4 + 4 x Binding post. Cooling; 4 x fans, airflow from front to back. Power Supply; 220V-230V 50/60Hz, 1500VA. Dimensions (W x D x H); 483mm (19”) x 452mm (17.8”) x 88mm (3.5”). Packaging Dimensions (1 piece/pack); 550mm (21.7”) x 540mm (21.3”) x 175mm (6.9”). Net Weight (1 piece/pack); 30kg (66.1lb). Gross Weight (1 piece/pack); 32kg (70.5lb). | Mouseover image to zoom Front View Back View Planform. |
| --- | --- |