U8

8” Two Way Full Range Plastic Speaker

| Product Description: Portable and installed plastic loudspeakers used in applications including sound reinforcement in Front of House and Foldback applications in Houses of Worship,Events, Clubs, Rehearsal Halls etc. The U Active Series delivers extraordinary performance at unmatched value. Features: - High output SPL and sensitivity - 1 x 8” high power transducer with 50mm voice coil - 1 x 1.3” diaphragm compression driver - 90º x 60º controlled coverage pattern - Rotatable Horn - Heavy duty protective grill - Injection molded - 1 x NL4 connector - Cabinet design for FOH and Monitor use - Suitable for use as a main system with or without sub speaker augmentation - Portable system (stand), or permanent installation (wall, ceiling, under balcony) - Arrays with all active high power U Series sub speakers for extended low frequency, high quality music reproduction - Monitor kit - optional Cabinet Dimension: Specifications. Construction; Plastic. Transducers; 1 x 1.3” HF + 1 x 8” LF. Frequency Response (-3dB); 65Hz-18kHz. Rated Power; 100W (RMS). Sensitivity (1W @ 1m); 92dB. Maximum SPL (1W @ 1m); 111dB (Peak). Rated Impedance; 8Ω. Dispersion (HxV); 90°x60°. Connector; 1xNL4. Dimensions (WxDxH); 277mm (10.9”) x 257mm (10.1”) x 400mm (15.7”). Packaging Dimensions (1 pair/pack); 595mm (23.4”) x 310mm (12.2”) x 600mm (23.6”). Net Weight (1 piece/pack); 6.5kg (14.3lb). Gross Weight (1 pair/pack); 15kg (33.1lb). Installation drawing: | Mouseover image to zoom. Relevant Download: U Series Catalog User Manual U8 Image. |
| --- | --- |