U6

6” Two Way Full Range Plastic Speaker

| Product Description: Portable and installed plastic loudspeakers used in applications including sound reinforcement in Front of House and Foldback applications in Houses of Worship,Events, Clubs, Rehearsal Halls etc. The U Active Series delivers extraordinary performance at unmatched value. Key Features and Benefits: - High output SPL and sensitivity - 1 x 6” high power transducer with 50mm voice coil - 1 x 1” diaphragm compression driver - 90º x 60º controlled coverage pattern - Rotatable Horn - Heavy duty protective grill - Injection molded - 1 x NL4 connector - Cabinet design for FOH and Monitor use - Suitable for use as a main system with or without sub speaker augmentation - Portable system (stand), or permanent installation (wall, ceiling, under balcony) - Arrays with all active high power U Series sub speakers for extended low frequency, high quality music reproduction - Monitor kit - optional Cabinet Dimension: Specifications. Construction; Plastic. Transducers; 1 x 1” HF + 1 x 6.5” LF. Frequency Response (-3dB); 80Hz-18kHz. Rated Power; 80W (RMS). Sensitivity (1W @ 1m); 91dB. Maximum SPL (1W @ 1m); 110dB (Peak). Rated Impedance; 8Ω. Dispersion (H x V ); 90° x 60°. Connector; 1 x NL4. Dimensions (W x D x H); 225mm (8.9”) x 211mm (8.3”) x 325mm (12.8”). Packaging Dimensions (1 pair/pack); 380mm (15”) x 255mm (10”) x 480mm (18.9”). Net Weight (1 piece/pack); 5.6kg (12.3lb). Gross Weight (1 pair/pack); 13.2kg (29.1lb). Installation drawing: | Mouseover image to zoom. Relevant Download: U Series Catalog User Manual U6 Image. |
| --- | --- |